In the first days of my internship at Movares, I discovered some main research terms that are used within the field of research that I am looking for. These are:

- Life cycle cost analysis (LCCA)

- Condition-based maintenance (CBM)

- Infrastructure asset management

- Policy improvement algorithm

- Treinvrije periodes (TVPs)

- Maintenance management

- Maintenance optimisation

- Number of conditioning states

  - Two states (functioning / failing)

  - Three states (good, defective, failed) also called delay-time model

  - Multiple discrete states

    - Discrete-time Markov chain

    - Continuous-time Markov chains

    - Semi-Markov chains

    - Proportional hazard model (PHM) with failure rate (depended on condition) (in which the failure rate is a function of both the age and the condition of equipment)

    - Production system

  - Continuous state space

- Maintenance actions

  - Preventive maintenance

    - Condition-based maintenance (CBM)

      - Conditions monitored by preforming inspections

        - Periodic

        - Aperiodic

      - Conditions monitored continuously

  - Corrective maintenance

- Replacement and repair

  - Repairable

    - Perfect repair (good as new)

    - Imperfect

    - Corrective repair

      - Minimal repair (returned to (just) functioning state)

  - Non-repairable

    - Maintenance = replacement

- Dependencies

  - Economic dependency

    - Maintaining/inspecting multiple assets at once

  - Structural dependency

    - Functioning is dependent? -> Not really the case as for

  - Stochastic dependency

    - Deterioration is dependent?

      - Assume that this is not the case, for simplification of the model. In reality, same train over track.

- Optimality criteria

  - Minimisation of long-run cost rate

  - Minimisation of total discounted costs during finite horizon

  - Maximisation of long run availability

  - Maximisation of reliability

Ontmoet Marc - Movares Magazine editie 1 2023

Based on these research terms, I was able to find the following papers:

- Strategic Maintenance Optimization in Infrastructure Networks - Research portal Eindhoven University of Technology Network wide maintenance cost reduction.

- https://journals.sagepub.com/doi/10.1177/1748006X17742765 Generic Bayesian network models for making maintenance decisions from available data and expert 

For a sufficient number of observations, Billingsley (1961) has shown that the maximum likelihood estimator for the transition probability from state i to j is simply given by ˆPij=Nij/ m j=0Nij,whereNij is thenumberofobservedtransitionsfromitoj.

Billingsley, P., 1961. Statistical Inference for Markov Processes. University of Chicago Press, Chicago.

DETAILS THAT SHOULD BE IN REPORT
Also describe dead ends, or process etc
Results: model, designs,

Too simple to relate to reality, and also the downsides

Graduate seminar every month on Thursday. Anne and Mathias organise it. MOR presents subjects there. 20 minutes.

Can give ideas on project, but might make sense to have a few meetings. Contact via email is very quickly.

Conversation with Kristian – Civil engineering expert

I asked Kristian about his knowledge and experience on bridges. I specifically asked him to judge the risks of different aspects of a bridge as mentioned in the ‘kunstwerkpaspoort’. He said that this was very difficult to do as this depends so heavily on the location and situation of certain bridges. This is an individual process for each asset. And even with historical availability of data on inspections, these are subjective to begin with. Things are very variable and can not really be quantified, even with drone data and the use of AI for example. Therefore, he based his scores on his expertise on how often issues in that category occurred.

Conversation with Marc  – Expert on the project  “Grip op Kunstwerken”

The conversation with Marc was mainly listening to the visions that Marc has on the question that has been posed. I asked him about the underlying risk scores that were given to assets in the PowerBI tool. He said that this looks at different risk scores, which were given as the top 10 risks. These have all been given a colour green, yellow or orange. Then these are added up to give a score and then multiplied by the ‘baanurgentie’.

Contact opnemen met:

- Tom Waarseveen: Stalen bruggen expert

- Margreet Gerdessen: Betonnen bruggen expert

- Jeroen van Lunteren: kostenexpert

- Marco Post: Database / dataset informatie

Conversation with Carla  – Expert on mathematical modelling of moving bridges

Carla and Fernão have build a mathematical model that determines the optimal way of moving a bridge up and down via the motor that minimizes the stress on all elements on the bridge. They did so by making a model that has bridge properties as input, with the whole system of elements and outputs the motor instructions that would be optimal using solver MIDACO.

This program can also be used in Python. Movares has a license for this solver. Therefore, it could be interesting. They have implemented MICADO using C sharp, but there are also experts using Python. I could contact Carla for any issues regarding python problems.

Nynke Brouwer knows a lot about data driven modelling, but she is away from work because of pregnancy. It would be nice for future work to talk to her if she returns before my internship finishes. Otherwise, direct colleagues of Nynke would also work.

Furthermore, she tipped to include research on possible solvers comparison. Using aspects such as  performance and scalability. Even if this is a +/- table in the final report. Compare for example; brute force, simulated annealing, differential evolution and MIDACO. And Gurobi??

Afkortingen LCC documenten:

LCC = Life Cycle Costs

MJOP = Meerjarig Onderhoudsplan

LTP = Lange Termijn Planning

PVE = Programma van Eisen

PGO = Prestatie Gericht Onderhoud

tlv = ten laste van

BHM = Beheer maatregel

IB = Ingenieursbureau

FMECA = Failure Modes, Effects, and Criticality Analysis

insp = inspectie

ko = klein onderhoud

go = groot onderhoud

vv = vervangen

xo = extra onderhoud

NCW = Netto Constante Waarde

TVP = Trein Vrije Periodes

PV-pad = pad langs het spoor, voor onderhoud/evacuatie??

m.n. = met name

obv = op basis van